8 Channels 8 PoE 1HDD Network Video Recorder

E-N-AI08P-1 PRO

Dimensions mm (inch)

Panels

Technical Specification

| System |  |
| --- | --- |
| Main Processor | Industrial-grade processor |
| Operating System | Embedded Linux |
| Operating Interface | Web, Local GUI |

| AI |  |
| --- | --- |
| AI by Recorder | SMD Plus |
| AI by Camera | Face detection; face recognition; perimeter protection; SMD Plus; people counting; stereo analysis; heat map |

| Perimeter Protection |  |
| --- | --- |
| Perimeter Performance of AI by Camera (Number of Channels) | 6 channels |

| Network |  |
| --- | --- |
| Network Protocol | HTTP; HTTPS; TCP/IP; IPv4; IPv6; UDP; NTP; DHCP; DNS; SMTP; UPnP; IP Filter; FTP; DDNS; SNMP; Alarm Server; P2P; Auto Registration; IP Search (Supports IP camera, DVR, NVS, etc.) |
| Mobile Phone Access | Android; iOS |
| Interoperability | ONVIF 22.12(Profile T; Profile S; Profile G); CGI; SDK |
| Browser | Chrome; IE; Safari; Edge; Firefox |
| Record Playback |  |
| Multi-channel Playback | Up to 8 channels |
| Record Mode | General, motion detection; intelligent; alarm |
| Backup | USB device and network |
| Playback Mode | Instant playback, general playback, event playback, tag playback, smart playback |
| Alarm |  |
| General Alarm | Motion detection; privacy masking; video loss; scene changing; PIR alarm; IPC external alarm |
| Anomaly Alarm | Camera disconnection; storage error; disk full; IP conflict; MAC conflict; login locked; and cybersecurity exception |
| Intelligent Alarm | Face detection; face recognition; perimeter protection; SMD Plus; people counting; stereo analysis; heat map |
| Alarm Linkage | Record; snapshot; IPC external alarm output; audio; buzzer; log; preset; email |

| Port |  |
| --- | --- |
| Audio Input | 1-channel RCA |
| Audio Output | 1-channel RCA |
| Disk Interface | 1 SATA port. Each disk can contain up to 20 TB. This limit |
| USB | 2 (2 rear USB 2.0 ports) |
| HDMI | 1 |
| VGA | 1 |
| Network Port | 1 (10/100/1000 Mbps Ethernet port, RJ-45) |
| PoE Port | 8 ports, 10/100 Mbps, IEEE 802.3 af/at |

| Face Detection |  |
| --- | --- |
| Face Attributes | Gender; age group; glasses; expressions; face mask; beard |
| Face Detection Performance of AI by Camera (Number of Channels) | 6 channels |

| Face Recognition |  |
| --- | --- |
| Face Recognition Performance of AI by Camera (Number of Channels) | 6 channels |

| SMD Plus |  |
| --- | --- |
| SMD Plus by Recorder | 4 channels: Secondary filtering for human and motor vehicle, reducing false alarms caused by leaves, rain and lighting condition change |
| SMD Plus by Camera | 6 channels |

| Audio and Video |  |
| --- | --- |
| Access Channel | 8 channels |
| Network Bandwidth | AI disabled: 160 Mbps incoming, 160 Mbps recording and 80 Mbps outgoing AI enabled: 80 Mbps incoming, 80 Mbps recording and 60 Mbps outgoing |
| Resolution | 12 MP; 8 MP; 6 MP; 5 MP; 4 MP; 3 MP; 1080p; 960p; 720p; D1; CIF; QCIF |
| Decoding Capability | AI disabled: 1-channel 12 MP@30 fps; 2-channel 8 MP@30 fps; 3-channel 5 MP@30 fps; 4-channel 4 MP@30 fps; 8-channel 1080p@30 fps AI enabled: 1-channel 12 MP@30 fps; 1-channel 8 MP@30 fps; 2-channel 5 MP@30 fps; 3-channel 4 MP@30 fps; 7-channel 1080p@30 fps |
| Video Output | 1 × VGA, 1 × HDMI Simultaneous: HDMI: 3840 × 2160, 1920 × 1080, 1280 × 1024, 1280 × 720 VGA: 1920 × 1080, 1280 × 1024, 1280 × 720 Heterogeneous: HDMI: 1920 × 1080, 1280 × 1024, 1280 × 720 VGA: 1920 × 1080, 1280 × 1024, 1280 × 720 |
| Multi-screen Display | Main Screen: 1/4/8/9 Sub Screen: 1/4/8/9 |
| Third Party Camera Access | ONVIF; Panasonic; Sony; Axis; Arecont; Pelco; Canon; Hanwha |

| Compression Standard |  |
| --- | --- |
| Video Compression | Smart H.265+; H.265; Smart H.264+; H.264 |
| Audio Compression | G.711a; G.711u; PCM; G726 |

Technical Specification

e

| General |  |
| --- | --- |
| Power Supply | 53 VDC, 1.81 A |
| Power Consumption | Total output of NVR is <10 W(without HDD) Total output power of PoE is 72 W, the maximum output power of a single port is 25.5 W |
| Net Weight | 0.69 kg |
| Gross Weight | 1.74 kg |
| Product Dimensions | 270.6 mm × 206.3 mm × 45.9 mm (W × L × H) |
| Packaging Dimensions | 427 mm × 96 mm × 262 mm (W × L × H) |
| Operating Temperature | -10 °C to +55 °C |
| Storage Temperature | -20 °C to +60 °C |
| Operating Humidity | 10%–93% (RH) |
| Installation | Desktop or Rack |
| Certifications | CE-EMC: EN 55032:2015+A1:2020; EN IEC 61000-3-2:2019+A1:2021; EN 61000-3-3:2013+A1:2019+A2:2021; EN 55035:2017+A11:2020; EN 50130-4:2011+A1:2014 CE-LVD: EN 62368-1:2014 |

Technical Specification